| ## Terence Walsh Pest Management. Licence No. 1752 ABN – 27 422 482 252 Phone : 0416910688 | - SYDNEY - BLUE MOUNTAINS - CENTRAL COAST - CENTRAL WEST | - NEWCASTLE - NORTH COAST - SOUTH COAST - SOUTHERN HIGHLANDS |
| --- | --- | --- |

# Subterranean Termite Treatment Proposal - AS 3660.2.2000

Specifications for termite colony control procedures

It is very important that you DO NOT disturb any termite workings, leads, galleries, etc before commencement of the treatment program. If you disturb the termites then these treatment procedures may be rendered ineffective. Where applicable: Suspect trees and stumps within 50 metres of the structures, usually only within the boundaries of the property, are test drilled using an auger drill to determine whether a termite colony is present in the vicinity. If a nest is located it is destroyed by the careful application of a suitable termiticide. After drilling, any living trees are treated with a plant protection compound to prevent any subsequent plant disease prior to the sealing of the hole. A termiticide dust is carefully applied to suitable termite workings and/or leads which must then be left undisturbed for a period of approximately 30 days before re-examination and evaluation. If termite activity is still present the termiticide dust will be reapplied and the property reinspected in a further month. Any liquid chemical application recommended should not be applied until all termite activity has ceased within the property. The amount of termiticide dust applied varies with the chemical used and the size of the infestation. Generally 10g or less is all that is required. Where a termite baiting program is recommended a suitable number of termite bait stations will be installed in the grounds of the property. These must be left undisturbed and will be checked by us on a routine basis. When a suitable level of termite attack has occurred a termite dusting treatment, as detailed above, will be performed. This procedure is dependent upon the termites finding the bait stations and may take several months or more, to be effective.

Specifications for treatment of accessible subfloor crawl spaces

Termiticide is applied at the rate of 5 litres per square metre for horizontal barriers up to 50 mm deep and 100 litres per cubic metre for vertical barriers so as to form a complete and continuous treated soil barrier (both vertical and horizontal) by trenching and rodding beneath and around the structures. Soil is loosened to a depth of at least 80mm and spacings for rodding are no more than 150 mm apart. The barrier is continuous where the subfloor crawl space is less than 400mm high; and at least 150mm wide, abutting all substructure piers, walls and connections, where the subfloor crawl space is more than 400mm high.

Specifications for drilling of concrete slab areas

Where concrete areas such as driveways, paths or the like are located in external areas that are to be treated, it is always our first recommendation to have the concrete cut and removed to allow full access for treatment. It is not possible to verify that a complete barrier has been installed when drilling and injecting termiticide beneath concrete. Concrete slabs and paths, both internally and externally will be drilled, using a concrete hammer drill. Holes around perimeter walls will be drilled in accord with AS 3660.2.2000 at up to 300 mm centres, depending upon the substrate beneath the concrete. Holes are drilled as close as possible to the walls. The termiticide is applied at a rate of 10 litres per lineal metre ie up to 2 litres per hole. Holes will be sealed on completion. Where a recommendation has been made to grid pattern drill a concrete slab, the perimeter of the slab will be drilled as above and the remainder of the slab drilled in a chequer board fashion. The hole centres will be drilled at up to 200mm intervals, in rows at up to 1 metre apart. Termiticide will be applied to the holes at a rate of up to 2 litres per hole. Holes will be sealed on completion.

Specifications for ventilation

The Australian Standard recommends that subfloor regions should have adequate cross flow of air equivalent to 7300 mm2 net ventilation area per lineal metre on external and internal walls.

IMPORTANT INFORMATION: The Australian Standard AS 3660 series details methods for the detection, treatment and minimisation of subterranean termite infestation in and around buildings. These barriers impede termites from gaining concealed access to timber and other termite susceptible material. AS 3660 also details methods for minimising the risk of reinfestation by termites.

| Client. | Structure at: |
| --- | --- |
| Address:. |  |
| State:; Postcode:. | Phone:. |
| Date of Proposal:. | Facsimile:. |
| Provided by:. | N; Client. Not Applicable; Accepts Y. Y; Price; Client. We recommend; Refuses N. |
| . |  |
| INSPECTION ANDCOLONY CONTROL MEASURES Perform thorough inspection with written report in accordance with AS 3660.2.2000 | Y. |
| A. Gain access to the following concealed areas: - |  |
| B. Check & test drill suspect trees, stumps and timber piers as detailed below: - |  |
| C. Control the termite colony by removal and/or direct destruction of the nest in the following Areas:-. |  |
| D. Treat the termite colony indirectly by applying dust to termite workings in the following areas: - |  |
| E. Implement a termite monitoring and/or baiting program in the following areas: - |  |
| F. Improve ventilation by: - |  |
| G. Remove materials conducive to termite infestation, eg timber retaining walls: - |  |
| H. You must arrange for a builder to determine the extent of any structural timber damage. |  |
| I. In our presence, you must arrange for a builder to expose termite entry points in the following areas: - |  |
| J. You must arrange for repair/installation of Termite shielding (Ant Capping) in the Following areas:- |  |
| K. You must arrange to improve drainage and reduce moisture levels in the following areas: - |  |
| L. A further inspection will be carried out in days to assess the above colony control measures. If termite activity is. found further colony control measures will be undertaken. Only when all termite activity appears to have ceased (evidence of live termite activity not found) within the inspected building/s will the barrier treatments commence. |
| BARRIER TREATMENT OPTIONS in accordance with AS 3660.2.2000 and the product label Accessible (suspended) subfloor areas |  |
| M. Where clearance is more than 400mm trench or scarify to create a vertical and horizontal barrier or treated zone around all substructure walls and piers with a minimum depth of 50mm below the top of the footings and a minimum width of 150mm. Termiticide will be applied in accord with AS 3660.2.2000 at the rate of 100 L per m3 with the soil being backfilled. |  |
| N. Where clearance is less than 400mm scarify to a depth of 50 mm and create a horizontal barrier over the entire subfloor soil surface (access permitting) by applying termiticide at 5 L per m2 as per AS 3660.2.2000. |  |

| N Client. Not Applicable; Accepts Y. Y; Price; Client. We recommend; Refuses N. Concrete slab on ground - inside the structure - |
| --- |
| O. Drill holes along the internal edge of the slab not more than 150 mm out from the wall at 150 mm to maximum of 300 mm centres and inject with termiticide at a rate of 1.5 L to 2 L per hole as per AS 3660.2.2000. Holes will be sealed on completion. |  |  |  |
| P. Drill holes along one side of all the internal walls not more than 150 mm out from the wall at 150 mm to maximum of 300 mm centres and inject with termiticide at a rate of 1.5 L to 2 L per hole as per AS 3660.2.2000. Holes will be sealed on completion. |  |  |  |
| Q. Other. |  |  |  |
| Paths, patios, pavers and steps – Where concrete, brick or paver patios, paths or steps abut the structure |  |  |  |
| R. We/You arrange to cut concrete and/or remove bricks or pavers to allow for a treatment strip of at least 150mm wide. |  |  |  |
| S. Trench to a depth of 50 mm below the top of the footing (minimum of 80 mm) and apply termiticide while back filling as per AS 3660 at the rate of 100 L per m3. |  |  |  |
| T. Drill holes in patios, verandah, steps or paths as close as possible to the adjacent external walls of the structure at 150 mm to a maximum of 200 mm centres and inject termiticide at a rate of 1.5 L to 2 L per hole as per AS 3660. Holes will be sealed. |  |  |  |
| U. Remove bricks as appropriate to view under all in-fill slabs. |  |  |  |
| V. If not filled then access will be cut and an application as detailed in Accessible (suspended) subfloor areas (see page 2) will be carried out. If filled then grid pattern drill all patios, steps, verandahs and all in-fill concrete slabs and inject termiticide to form a continuous horizontal barrier. Holes will be sealed. |  |  |  |
| W. Other. |  |  |  |
| X. Trench along all external walls of the building down 50 mm below the top of the footing. Apply termiticide and back fill as per AS 3660 at the rate of 100 L per m3 .The minimum depth of this barrier must not be less than 80 mm. |  |  |  |
| Y. Other. |  |  |  |
| Follow up inspections - |  |  |  |
| Following the completion of the above treatment program, an inspection will be carried out in; days and then again at 6 months after the original treatment.. | Y. |  |  |
| Subtotal of all work agreed by you as marked above: - If you accept ALL of the above recommendations, See Important Conditions on page 4 we will provide a; month Free Service Period. (see Page 5):-. |  |  |  |
|  | SUB |  |  |
| Client’s Acceptance: ______________________________________________ | GST |  |  |
| Termiticide/s to be used:. | Total |  |  |

| 2. If you DO NOT accept all of the above recommendations you will |  |  |  |
| --- | --- | --- | --- |
| receive a free service period for; months. However the free service. |  |  |  |
| period only applies to the work you agreed to have performed. Areas NOT treated will NOT be covered. | SUB |  |  |
| by the free service period and will only be treated at your expense. See Important Conditions on page 4 | GST |  |  |
| Client’s Acceptance: ______________________________________________. | Total |  |  |
| Termiticide/s to be used:. |
| 3. Some building construction makes the installation of a complete termite barrier impossible without carrying out major building work. Your property is such a building. We CANNOT warrant in any way that the treatment proposed will result in completely excluding termite entry. See Important Conditions on page 4 | SUB GST |  |  |
| Client’s Acceptance: ______________________________________________. | Total |  |  |
| Termiticide/s to be used:. |
| Specific Treatment Limitations - |  |  |  |
| Regular Inspections - Termite barriers do not kill termites, they only impede concealed entry thus forcing the termites to show themselves and making them easier to find during regular inspections. The Australian Standard AS 3660 recommends “regular, competent inspections should be carried out at least on an annual basis, but more frequent inspections are strongly recommended”. |  |  |  |
| An inspection program of; 1; Inspections per year should commence not later than six (6) months after the last ‘follow up. |
| inspection’ as recommended. This inspection will be in accordance with AS 3660 and a full report will be provided. |  |  |  |
| Additional information: |  |  |  |
| Free Service Period: Where a FREE Service Period has been provided then, upon notification by you, we agree to provide you with any remedial treatment(s) as may be required to the treated structures, AT NO COST TO YOU. Any FREE Service Period is conditional upon you notifying us of signs of termite infestation. Fences, gates, pergolas, and garden retaining walls are excluded from the terms of any FREE Service Period. Any signs of activity in these areas must be reported to us. © Copyright 1996 RAPID Solutions |  |  |  |

Important Conditions concerning your Subterranean Termite Treatment

1. Warning. Where drilling and/or cutting is required through brick, concrete, timber or other surfaces no liability can be accepted should damage result to concealed services such as power, gas, water, etc. You should provide clear and accurate plans before commencement. You indemnify us against any costs that may arise from such possible damages if plans are not provided.

2. If during the course of the treatment program it is found that structural or environmental conditions prevent us performing any sections of the agreed Treatment Program then the Free service period offered may have to be reviewed or the cost of your investment may have to be revised. The prices advised overleaf are valid for 30 days, after that we reserve the right to make revisions.

3. All monies are due and payable upon invoice at the commencement of the treatment program. All free service periods are null and void unless payment has been received in full.

4. You will arrange for free access to the property. Any restrictions to access or delays in granting access may result in us revising the investment required for treatment. You will provide 240v AC 15 amp power supply within 30 metres of any areas to be drilled.

5. This proposal only applies to the treatment and application of barrier/s to the structure(s) as detailed in this contract against attack by subterranean termites. It does not provide for protection against any other pest/s. In particular it does not include protection against “drywood termites”, FAMILY: KALOTERMITIDAE.

6. In the event of subterranean termites returning within the service period, and upon notification by you, we will provide such services as detailed in that contract as accepted by you. Any free service is conditional upon you notifying us of any signs of termite infestation. Fences, gates, pergolas, and garden retaining walls are specifically excluded from the terms of any service period. However, any signs of activity in these areas or in any part of the property must always be reported to us within 7 days.

7. No responsibility is accepted, or warranty implied, for any timber damage that may occur as the result of past, current or future termite activity.

8. This Agreement may be extended at the end of the free service period, at our discretion, subject to you paying a further maintenance fee to be notified, and subject to our completing an inspection report and treatment proposal and you accepting our recommendations as notified in that report. Modern termite control agents are designed to be biodegradable therefore, the life of the barrier/s they provide is limited by various environmental factors. The barrier will need to be renewed when the normal life expectancy (related to diminishing residual strength of chemical over time and environmental factors) of this barrier is considered to have diminished and may not provide the level of protection as indicated on the product label. It is therefore imperative that you have regular, competent inspections by a licensed pest management firm (at least annually), and retreatments, as determined by those inspections.

9. You accept that this termite treatment program can be rendered ineffective due to you making building alterations, renovations, additions (including pergolas, awnings, verandahs, etc), introducing conducive materials, disturbing external gardens, pathways, etc adjacent to the areas protected and through establishing lawns &/or garden beds adjacent to the protected areas. (Such changes to the property are likely to breach the termite barrier). Where such changes occur you must contact us for further treatment advice. You must be careful and take precautions to ensure that you do not in any way damage the soil barrier created.

10. With a concrete slab on ground home it is important that you ensure the edge of the slab remains exposed and is not covered up by garden materials eg soil, pine bark or similar. Also ensure that air vents or weep holes are never blocked.

11. Do not use untreated timbers for garden beds or retaining walls, as they may attract termites.

12. Warning. Where drilling and or cutting is required through brick, concrete, timber or other surfaces no liability can be accepted for the resulting damage to floor tiles, carpet, any other floor coverings or to the brick, concrete, timber or other surfaces.

13. No liability or responsibility is accepted for termite entry to treated areas resulting from poor building design, or where the construction does not comply with AS 3660.1-2000 and the Building Code of Australia. Examples of such situations include, but are not limited to, cracking of concrete slabs, inadequate waterproofing, covering weep holes, inadequate termite protection to service entry points through concrete slabs, and the failure to fully remove formwork timbers. Such faults may not be visible and as a result are not noted as limitations to the treatment.

14. It is not always possible to fully protect a structure from termite attack without major structural alterations. See the diagram “Some Limitations To The Treatment” on page 6. This illustrates some common ways that termites can enter a building even after a termite barrier treatment has been installed.

Visual Timber Pest Inspection & Report in accord with AS 4349.3

| Account to: |  |  |  |
| --- | --- | --- | --- |
| Phone: |  | FAX: |  |
| Client: |  | report No: |  |
| Purchaser: |  | Vendor: |  |
| Re: Structure at: |  |  |  |
| Date of the Inspection: |  | file no: |  |

Summary Only.                IMPORTANT DISCLAIMER.

- This Summary is supplied to allow a quick and superficial overview of the inspection results.

- This Summary is NOT the Report and cannot be relied upon on its own.

- This Summary must be read in conjunction with the full report and not in isolation from the report.

- If there should happen to be any discrepancy between anything in the Report and anything in this Summary, the information in the Report shall override that in this Summary.

- ACCESS:

Are there any Area(s) and/or Section(s) to which Access should be gained:

TIMBER PEST ACTIVITY:

Were active subterranean termites (live specimens) found:

See Clause 3, 4 , 5 & 6 Page 8

Was visible evidence of subterranean termite workings or damage found:

See Clause 8 Page 10 and Clauses 3, 4 & 5 Page 8

Was visible evidence of possible subterranean termite workings found:

See Clause 8A Page 10

Was visible evidence of borers of seasoned timbers found:

See Clause 16 Page 12

Was evidence of damage caused by wood decay (rot) fungi found:

See Clause 17 Page 12

Was visible evidence of a possible or previous subterranean termite treatment found:

See Clause 10 Page 10 and Clauses 3, 4 and 5 Page 8

For complete and accurate information please refer to the attached complete

Visual Timber Pest Report, provided in accordance with AS 4349. 3

Visual Timber Pest Report:

Important Information: Any person who relies upon the contents of this report does so acknowledging that the                                                                                        following clauses which define the Scope and Limitations of the inspection form an integral part of the report.

1.	THIS IS A VISUAL INSPECTION ONLY in accord with the requirements of AS 4349.3 Inspection of buildings Part 3: Timber pest inspections. Visual inspection was limited to those areas and sections of the property to which reasonable access (See definition on page 6 of this report) was both available and permitted on the date of Inspection. The inspection DID NOT include breaking apart, dismantling, removing or moving objects including, but not limited to, foliage, mouldings, roof insulation/sisalation, floor or wall coverings, sidings, ceilings, floors, furnishings, appliances or personal possessions. The inspector DID NOT  lift the carpet to visually inspect the underlay, floorboards and no guarantee is implied that such area is free of termite activity and/or damage. SUCH DAMAGE OR ACTIVITY MAY ONLY BE NOTICED WHEN CARPET IS LIFTED. WE STRONGLY RECOMMEND THAT CARPET BE LIFTED FOR FULL VISUAL INSPECTION. FAILURE TO COMPLY WITH THIS REQUIREMENT COULD EXPOSE THE HOUSE TO POSSIBLE EXTENSIVE DAMAGE BY UNDETECTED TERMITE ACTIVITY. The inspector CANNOT see inside walls, between floors, inside skillion roofing, inside the eaves, behind stored goods in cupboards, in other areas that are concealed or obstructed. The inspector DID NOT dig, gouge, force or perform any other invasive procedures. An invasive inspection will not be performed unless a separate contract is entered into. In an occupied property it must be understood that furnishings or household items may be concealing evidence of Timber Pests (white ants) which may only be revealed when the items are moved or removed.

2.	SCOPE OF REPORT. This Report is confined to reporting on the discovery, or non discovery, of infestation and/or damage caused by subterranean and dampwood termites (white ants), borers of seasoned timber and wood decay fungi (hereinafter referred to as "Timber Pests"), present on the date of the Inspection. The Inspection did not cover any other pests and this Report does not comment on them. Dry wood termites (Family: KALOTERMITIDAE) were excluded from the Inspection, but have been reported on if, in the course of the Inspection, any visual evidence of infestation happened to be found.

3.	LIMITATIONS. Nothing contained in the Report implies that any inaccessible or partly inaccessible areas or sections of the property being inspected by the Inspector on the date of the Inspection were not, or have not been, infested by Timber Pests. Accordingly this Report is not a guarantee that an infestation and/or damage does not exist in any inaccessible or partly inaccessible areas or sections of the property. Nor is it a guarantee that a future infestation of Timber Pests will not occur or be found.

4.	DETERMINING Extent of damage. This Report does not and cannot state the extent of damage. It is NOT a structural damage report. We claim no expertise in structural engineering. If any evidence of Timber Pest activity or damage is Reported, then it must be assumed there may be some structural damage and a qualified person such as a Builder, Engineer, Architect or other qualified expert in the COTTAGE CONSTRUCTION INDUSTRY BE CONSULTED to determine the extent of the damage, if any and the extent of repairs that may be required. This firm is not responsible for the repair of any damage whether disclosed by this report or not.

5.	POSSIBLE HIDDEN DAMAGE. If Timber Pest activity and/or damage is found, or evidence of a previous possible treatment is found, within the Structures or the grounds of the property, then damage may exist in concealed areas, eg framing timbers. An INVASIVE INSPECTION is strongly recommended in this case. Damage may only be found when wall linings, cladding or insulation are removed to reveal previously concealed timbers.

6.	CONSUMER COMPLAINTS PROCEDURE. In the event of any controversy or claim arising out of, or relating to this Timber Pest Property Report, either party must give written Notice of the dispute to the other party. If the dispute is not resolved within ten (10) days from the service of the Notice then the dispute shall be referred to a mediator nominated by the Inspector. Should the dispute not be resolved by mediation then either party may refer the dispute to the Institute of Arbitrators and Mediators of Australia for resolution by arbitration.

1)   Brief Description of Structure(s) Inspected:

| TYPE |  |
| --- | --- |
| HEIGHT |  |
| BUILT FROM |  |
| PIERS TYPE |  |
| ROOFING |  |
| FLOORING |  |
| FENCES |  |

When a building, or part of a building is constructed on a concrete slab it is always more susceptible to concealed termite entry. An invasive timber pest inspection should always be carried out when the house is constructed on a concrete slab. All documentation relating to previous termite management programmes be supplied by the vendor. Failure to comply with this recommendation could expose the house to possible future termite infestation, causing possible future structural damage.

2)	Areas Inspected:

3)  Other Area(s)* to which REASONABLE ACCESS for Inspection was NOT AVAILABLE

and the Reason(s) why include:

3.1) SUBFLOOR BECAUSE OF SLAB CONSTRUCTION

* Please note since complete inspection of the above areas was not possible, timber pest activity and/or damage may exist in these areas.

No inspection was made, and no report is submitted, of inaccessible areas. These include, but may not be limited to, concealed frame timbers, eaves, areas concealed by concrete floors, wall linings, soil, landscaping, rubbish, floor coverings, furniture, pictures, appliances, stored items, insulation, hollow blocks/posts, etc. Furnishings, furniture & stored items were not inspected.

4)	Area(s)* in which Visual Inspection was Obstructed or Restricted and the Reason(s) why include:

* Please note since complete inspection of the above areas was not possible, timber pest activity and/or damage may exist in these areas.

- High Risk Area(s) to which Access should be gained, or fully gained, since they may show evidence of Timber Pests or damage:

Was the property furnished at the time of inspection? YES

Where a property is furnished at the time of the inspection then you must understand that the furnishings and stored goods may be concealing evidence of Timber Pest Activity. This evidence may only be revealed when the property is vacated. A further inspection of the vacant property is strongly recommended in this case.

6)   SUBTERRANEAN TERMITES:

Were active termites (live insects) present at the time of inspection? NO

7)      The termites are believed to be:

8)     Visible evidence of subterranean termite workings and/or

damage was found:

BATHROOM OF WORKERS COTTAGE

VERY Important: Where any termite activity or damage is noted above you must realise that further termite damage may be present in concealed areas. See Clauses 3, 4 and 5 on page 3.

8a) VISIBLE EVIDENCE OF POSSIBLE termite workings and/or damage was

found:

* VERY IMPORTANT. (See Clause  3, 4, and 5 on page 3)

9)   Whilst we are not builders, the termite damage appears to be:

IMPORTANT:   Where  visual evidence  of termite  workings and/or  damage is reported above,  but no live  termites were present at the time of inspection, you must realise that it is possible that termites are still active  in the immediate vicinity  and the termites may continue to  cause further  damage. It is not  possible, without   benefit of further investigation and inspections over a period of time, to  ascertain whether any infestation is  active or inactive. Active termites  may simply have not  been present  at the time of  inspection due to a prior disturbance, climatic  conditions,  or  they may have been  utilising  an alternative feeding source. Continued, regular, inspections are essential. Unless  written  evidence  of a termite  protection  program  in  accord with  "Australian Standard  3660. 2/ 2000 - Protection  of  buildings  from subterranean termites"  is provided, a treatment should always be considered to reduce the risk of further attack.

- The following evidence of a possible previous termite treatment was found:  DRILL HOLES TO WORKERS COTTAGE

This firm can give no assurances with regard to work that may have been previously performed by other firms. The firm which treated the property must be contacted for treatment and warranty information. In many cases retreatment may be required. Subterranean termite treatment recommendation: A management program in accordance with AS 3660. 2/ 2000 to protect against subterranean termites is considered to be essential and regular inspections are imperative. Termite Shields (Ant Caps) should be in good order and condition so termite mud tubes are exposed and visible. This helps stop termites gaining undetected entry. Missing, damaged or poor shields increase the risk of infestation.

- Was a durable sign located in the meter box indicating a

barrier  system  has been installed?

If a durable sign was located you should insist that the vendor supply all documentation

relating to the termite barrier system installation. These should include :

- A termite inspection certificate in accordance with Australian standards as 3660/1/2000 if the treatment was done as a construction requirement. Complete with all warranty documentation.

1a) A termite inspection certificate in accordance with Australian standards as 3660/2/2000 if

the treatment  was done as a result of a termite infestation.

- A certificate of application of a termite barrier in accordance with Australian standards 3660/2/2000. If these documents are not produced then we would recommend and insist that the house be treated in accordance with these specifications. Failure to comply with this recommendation could expose the house to possible future termite activity causing possible future extensive damage.

12) SUBTERRANEAN TERMITE TREATMENT RECOMMENDATION:

A management program in accordance with AS 3660.2.2000 to protect against subterranean   termites is considered to be essential.

- termite shields and physical barriers (ant caps etc):  should be in good order and condition so termite workings are exposed and visible.  This helps stop termites gaining undetected entry.  Joins in the shielding should have been soldered during the installation.  Whenever it is observed that joins in the shielding have not been soldered then the shielding must be reported as inadequate.  It may be possible for a builder to repair the shielding or a chemical barrier may need to be installed to provide a barrier to replace the use of shielding.  Missing, damaged or poor shields increase the risk of infestation. Whilst not a builder, it appears that termite shields are generally :

If considered inadequate a licensed builder or other expert in the cottage construction industry should be consulted.

N.B. Physical barrier systems are not visible to inspection and no comment is made on such systems.

- Other      areas    and/or    situations    that    appear    conducive    to

(may  attract)    subterranean   termite   infestation  are :

WET AREA AT REAR OF WORKERS COTTAGE

15)   at  the time of inspection the degree of risk of subterranean termite infestation to the overall property was considered to be high.

FUTURE INSPECTIONS:  Due to the degree of high risk of Subterranean termite infestation noted above, and all other findings of this report, we strongly recommend that a full inspection and written report in accord with AS 4349.3 or AS 3660.2-2000 is conducted at this property every.

AS 3660.2-2000 recommends “regular competent inspections should be carried out at least on an annual basis but more frequent inspections are strongly recommended”.  It goes on to inform that “regular inspections will not prevent termite attack, but may help in the detection of termite activity.  Early detection will allow remedial treatment to be commenced sooner and damage to be minimized”.

16) EVIDENCE OF BORER DAMAG/ACTIVITY :

THE SPECIES OF BORER IS BELIEVED TO BE

Note: Lyctus brunneus (powderpost beetle) is not considered a significant pest of timber. Damage is confined to  the sapwood  so treatment  or timber  replacement  is  not usually required. Unless  proof  of treatment is  provided,  Anobium  punctatum (furniture beetle) and Calymmaderus incisus (Queensland pine beetle) must  always be  considered active, because, unless  the timber  is ground up, one  cannot determine  conclusively if activity has ceased. Treatment and  timber replacement is required.

Borer  activity is usually  determined by the  presence of  exit holes and/or frass. Since a delay exists between the  time of  initial infestation and  the appearance of  these signs, it is possible  that some borer  activity may exist that is  not discernible  at the time  of inspection.

Borer  treatment  recommendations: Replacement of affected timbers is always preferred since, in the event of selling the property in the future it is probable that an  inspector  will report  the  borers  as active (see above). A chemical  treatment to  control and/or  protect  against  furniture  beetle  and/or  Queensland  pine beetle  can  be  considered  as a  less  effective, lower cost option. Before  considering  this  option  though  you  should consult  with a  builder  (see page 4, clause 4) to determine  if  the  timbers   are  structurally  sound. Following  the initial  treatment  a further  inspection  is  essential in twelve months time to determine if further treatment is needed.

17) FUNGAL DECAY CAUSED BY WOOD DECAY FUNGI:

Evidence of damage caused by past and/or present wood decay fungi (wet rot) was:

The damage appears to be:

17A)  Mould

Mildew and non wood decay fungi is commonly known as Mould and is not considered a Timber Pest. However, Mould and their spores may cause health problems or allergic reactions such as asthma and dermatitis in some people. No inspection for mould was carried out at the property and no report on the presence or absence of Mould is provided.

If Mould is noted as present within the property and you are concerned as to the possible health risk, resulting from its presence then you should seek advice from your local council, state or Commonwealth Government Health Department or a qualified expert such as an Industry Hygienist.

18)  ENVIRONMENTAL CONDITIONS THAT ARE CONDUCIVE TO TERMITES:

- Drainage:

Poor drainage, especially in or into the subfloor or against the external walls, increases the

likelihood of termite attack.  Whilst not a plumber, it appears that drainage is generally:

Areas where drainage should be attended to by a plumber or other expert:

REAR OF COTTAGE

B)  Water leaks:  Water leaks, especially in or into the subfloor or against the external walls,

increases the likelihood of termite attack.  Leaking showers or leaks from other ‘wet areas’also

increase the likelihood of concealed termite attack.

Whilst not a plumber, it appears that leaks are:

Areas where leaks should be attended to by a plumber or other expert:

REAR OF COTTAGE

Where drainage is considered inadequate or water leaks are reported then a plumber, builder or

other building  expert in the cottage construction industry should be consulted, as to cause and

extent of possible damage.

C)  Ventilation:

Ventilation, particularly to the sub-floor region is important in minimising the opportunity for

termites to establish themselves within a property. Whilst not a builder the ventilation appears

to be generally:

Where ventilation needs to be improved consult a builder or other expert  in the cottage

construction industry should be consulted as to cause and extent of damage.

- Slab Edge Exposure: Some types of slab on ground construction leave the slab edge exposed.

In many cases the edge forms part of the termite barrier.  Where external concrete slab edges

are not exposed there is a high risk of concealed termite entry.  Concrete paths, patios, pavers,

garden beds, etc often conceal these slab edges.

Whilst not a builder, in my opinion, at the time of construction the slab edge was intended as a

termite barrier Where the slab edge may form part of the barrier. The slab edge is intended to

be fully exposed on all sides of the building/s.

Was the slab edge exposed?

If the slab edge is not exposed, a licensed builder or other expert in the cottage construction

industry should be engaged to rectify the problem, so that concealed entry points for termites

via weep holes is visually noticed. It may be possible to install a termite barrier instead of

exposing the slab edge.  Please ask the inspector to advise you on this issue.

- Weep holes in external walls:  It is very important that soil, lawn, concrete paths or pavers do

not cover the weep holes.  They should be clean and free flowing.  Covering the weep holes in

part or in whole may allow undetected termite entry.

Were the weep holes in external walls covered?

If the weep holes were full or partially covered, it is imperative that they be uncovered. If  this is

not done you may expose the house to possible future subterranean termite activity causing possible

future extensive damage.

19) Comments of environmental condition:

20) Recommendations:

N.B. - Failure to comply with these directions could expose the house to possible future termite activity causing possible future extensive damage.

this company has gone to great lengths  to emphasize   the importance of being aware of the risk of termite infestation, that occur to houses in N.S.W. The importance of regular inspections and protEctinG your home in accordance with australian standards specification a.s. 3660\2\2000 SHOULD NOT BE IGNORED.

FAILURE TO COMPLY WITH ABOVE RECOMMENDATIONS COULD EXPOSE THE HOUSE TO POSSIBLE FUTURE TERMITE ACTIVITY CAUSING POSSIBLE FUTURE EXTENSIVE TERMITE DAMAGE.

General remarks: A more thorough INVASIVE INSPECTION is recommended, where any current visible evidence of Timber Pest activity is found. It is strongly recommended that a more invasive inspection is performed. Trees on the property up to a height of 2 metres and to a distance of up to 50 metres have been visually inspected, where possible and practicable, for evidence of termite activity. It is very difficult, and generally impossible to locate termite nests in trees since they are underground and evidence in trees is usually well concealed. We therefore strongly recommend that you arrange to have trees test drilled for evidence of termite nests or activity.

Important Maintenance Advice regarding Integrated Pest Management for Protecting against Timber Pests:

Any structure can be attacked by Timber Pests. Periodic maintenance should include measures to minimise possibilities of infestation in and around a property. Factors which may lead to infestation from Timber Pests include situations where the edge of the concrete slab is covered by soil or garden debris, filled areas, areas with less than 400mm clearance, foam insulation at foundations, earth/wood contact, damp areas, leaking pipes, etc; form-work timbers, scrap timber, tree stumps, mulch, tree branches touching the structure, wood rot, etc. Gardens, pathways or turf abutting or concealing the edge of a concrete slab will allow for concealed entry by timber pests. Any timber in contact with soil such as form-work, scrap timbers or stumps must be removed from under and around the buildings and any leaks repaired.

You should endeavour to ensure such conditions DO NOT occur around your property.

It is strongly recommended that a full Inspection and Report in accordance with Australian Standards AS4349.3 or AS3660.2-2000 be carried out every three months.  Regular inspections DO NOT stop timber pest attack, but are designed to limit the amount of damage that may occur by detecting problems early.

We further advise that you engage a professional pest control firm to provide a termite management program in accord with AS 3660/2/2000 to minimise the risk of termite attack. There is no way of preventing termite attack. Even AS 3660/2/2000 advises that “the provision of a complete termite barrier will impede and discourage termite entry into a building. It cannot prevent termite attack. Termites can still bridge or breach barriers but they can be detected more readily during routine inspections.”

DISCLAIMER OF LIABILITY: No liability shall be accepted on account of failure of the Report to notify any Termite activity and/or damage present at or prior to the date of the Report in any areas(s) or section(s) of the subject property physically inaccessible for inspection, or to which access for Inspection is denied by or to the Licensed Inspector (including but not limited to any area(s) or section(s) so specified by the Report).

DISCLAIMER OF LIABILITY TO THIRD PARTIES: This Report is made solely for the use and benefit of the Client named on the front of this report. No liability or responsibility whatsoever, in contract or tort, is accepted to any third party who may rely on the Report wholly or in part. Any third party acting or relying on this Report, in whole or in part, does so at their own risk.

# BEFORE you decide to purchase this property you should read and understand the following important information. It will help explain what is involved in a timber pest inspection, the difficulties faced by a timber pest inspector and why it is not possible to guarantee that a property is free of timber pests. It also details important information about what you can do to help protect your property from timber pests. This information forms an integral part of the report.

# REASONABLE ACCESS:

Only areas to which reasonable access is available were inspected. The Australian Standard 4349.3 defines reasonable access as “areas where safe, unobstructed access is provided and the minimum clearances specified in the Table below are available or, where these clearances are not available, areas within the consultant’s unobstructed line of sight and within arm’s length. Reasonable access does not include removing screws and bolts to access covers.” Reasonable access does not include the use of destructive or invasive inspection methods. Nor does reasonable access include cutting or making access traps, or moving heavy furniture or stored goods.

| Area | Access hole | Crawl space | Height |
| --- | --- | --- | --- |
| Roof interior | 450 x 400mm | 600 x 600mm | Accessible from 2.1m step ladder or 3.6m ladder placed against a wall. |
| Subfloor | 500 x 400mm | Vertical clearance: Timber floor: 400mm to bearer, joist or other obstruction. Concrete floor : 500mm |  |
| Roof Exterior |  |  | Accessible from a 3.6m ladder. |

# A MORE INVASIVE PHYSICAL INSPECTION IS RECOMMENDED:

As detailed there are many limitations to this visual inspection. Only with the permission of the owner of the premises you should engage a licensed building or other expert in the cottage construction industry to perform a more invasive physical inspection that involves moving or lifting: insulation, stored items, furniture or foliage during the inspection. We WILL physically touch, tap, test and when necessary force/gouge suspected accessible timbers and gain access to areas, where physically possible and considered practical and necessary, a licensed builder and/or other expert in the cottage construction industry be engaged to cut traps and access holes. This style of report is available by ordering with several days notice. Inspection time for this style of report will be greater than for a VISUAL INSPECTION. It involves disruption in the case of an occupied property, and some permanent marking is likely. You must arrange for the written permission of the owner who must acknowledge all the above information and confirm that our firm will not be held liable for any damage caused to the property. Price available on request.

# CONCRETE SLAB HOMES:

Homes constructed on concrete slabs pose special problems with respect to termite attack. If the edge of the slab is concealed by concrete paths, patios, pavers, garden beds, lawns, foliage, etc then it is possible for termites to gain concealed entry into the property. They can then cause extensive damage to concealed framing timbers. Even the most experienced inspector may be unable to detect their presence due to concealment by wall linings. Only when the termites attack timbers in the roof void, which may in turn be concealed by insulation, can their presence be detected. Where termite damage is located in the roof it should be expected that concealed framing timbers will be extensively damaged.

With a concrete slab home it is imperative that you expose the edge of the slab and ensure that foliage and garden beds do not cover the slab edge. Weep holes must be kept free of obstructions. All carpet edges are to be lifted to expose possible termite activity and/or concealed routes of entry available to termite entry and attack.

# SUBTERRANEAN TERMITES:

No property is safe from termites! Termites are the cause of the greatest economic losses of timber in service in NSW. Independent data compiled by CSIRO shows 1 in every 5 homes is attacked by termites at some stage in its life. Australia’s subterranean termite species (white ants) are the most destructive timber pests in the world. In fact it can take “as little as 3 months for a termite colony to severely damage almost all the timber in a home”.

How Termites Attack your Home. The most destructive species live in large underground nests containing several million timber destroying insects. The problem arises when a nest matures near your home. Your home provides natural shelter and a food source for the termites. The gallery system of a single colony may exploit food sources over as much as one hectare, with individual galleries extending up to 50 metres to enter your home, where there is a smorgasbord of timber to feast upon. Even concrete slabs do not act as a barrier; they can penetrate through cracks in the slab to gain access to your home. They even build mud tubes to gain access to above ground timbers. In some cases termites may create their nest in the cavity wall of the property without making ground contact. In these cases it may be impossible to determine their presence until extensive timber damage occurs.

Termite Damage. Once in contact with the timber they excavate it often leaving only a thin veneer on the outside. If left undiscovered these termites can cause many thousands of dollars damage and cost two to five thousand dollars (or more) to treat.

Subterranean Termite Ecology. These termites are social insects usually living in underground nests. Nests may be in trees or in rare instances they may be in above ground areas within the property. They tunnel underground to enter the building and then remain hidden within the timber making it very difficult to locate them. Where timbers are concealed, as in most modern homes, it makes it even more difficult to locate their presence. Especially if gardens have been built up around the home and termite barriers are either not in place or poorly maintained. Termites form nests in all sorts of locations and they are usually not visible. There may be more than one nest on a property. The diet of termites in the natural environment is the various hardwood and softwood species growing throughout Australia. These same timbers are used in buildings. Worker termites move out from their underground nest into surrounding areas where they obtain food and return to nurture the other casts of termites within the nest. Termites are extremely sensitive to temperature, humidity and light and hence cannot move over ground like most insects. They travel in mud encrusted tunnels to the source of food. Detection of termites is usually by locating these mud tunnels rising from the ground into the affected structure.

This takes an experienced timber pest inspector. You should insist that your timber pest inspector is a licensed pest manager with experience in termite eradication.

Termite Barriers

Termite barriers protect a building by forcing termites to show themselves. Termites can build mud tunnels around termite barriers to reach the timber above. The presence of termite tracks or leads does not necessarily mean that termites have entered the timber though. A clear view of walls and piers and easy access to the sub-floor is therefore imperative. However many styles of construction do not lend themselves to ready detection of termites. The design of some properties is such that they make the detection by a pest inspector difficult, if not impossible.

The tapping and probing of walls and internal timbers is an adjunct or additional means of detection of termites but is not as reliable as locating tracks. The use of a moisture meter is a useful aid for determining the presence of termites concealed behind thin wall panels, but it only detects high levels of activity. Older damage that has dried out will not be recorded. It may also provide false readings. Termite tracks may be present in the ceiling space however some roofs of a low pitch and with the presence of sisalation, insulation, air conditioning ductwork and hot water services may prevent a full inspection of the timbers in these areas.

Therefore the use of a moisture meter is a useful aid for determining the presence of termites concealed behind thin wall panels, but it only detects high levels since foolproof and absolute certain detection is not possible, the use of protective barriers and regular inspections is a very necessary step in protecting timbers from termite attack.

Borers of Seasoned Timbers:

Borers are the larvae of various species of beetles. The adult beetles lay their eggs within the timber. The eggs hatch out into larvae (grubs) which bore through the timber and can cause significant structural damage. The larvae may reside totally concealed within the timber for a period of several years before passing into a dormant pupal stage. Within the pupal case they metamorphose (change) into the adult beetle which cuts a hole in the outer surface of the timber to emerge, mate and lay further eggs to continue the cycle. It is only through the presence of these emergence holes, and the frass formed when the beetles cut the exit holes that their presence can be detected. Where floors are covered by carpets, tiling, or other floor coverings and where no access to the underfloor area is available it is not possible to determine whether borers are present or not. This is particularly the case with the upper floors of a dwelling.

Borers of ‘green’ unseasoned timber may also be present. However these species will naturally die out as the timbers dry out in service. Whilst some emergence holes may occur in a new property it would be unusual for such a borer to cause structural damage, though the exit holes may be unsightly.

Anobium borer (furniture beetle) and Queensland pine borer.

These beetles are responsible for instances of flooring collapse, often triggered by a heavy object being placed on the floor (or a person stepping on the affected area!) Pine timbers are favoured by this beetle and, while the sapwood is preferred, the heartwood is also sometimes attacked. Attack by this beetle is usually observed in timbers that have been in service for 10-20 years or more and mostly involves flooring and timber wall panelling. The frass from the flight holes (faeces and chewed wood) is fine and gritty. Wood attacked by these borers is often honeycombed.

Lyctus borer (powderpost beetle).

These borers only attack the sapwood of certain susceptible species of hardwood timber. Since it is a requirement that structural timbers contain no more than 25% Lyctus susceptible sapwood these borers are not normally associated with structural damage. Replacement of affected timbers is not recommended and treatment is not approved. Where decorative timbers are affected the emergence holes may be considered unsightly in which case timber replacement is the only option. Powderpost beetles mostly attack during the first 6-12 months of service life of timber. As only the sapwood is destroyed, larger dimensional timbers (such as rafters, bearers and joists) in a house are seldom weakened significantly to cause collapse. In small dimensional timbers (such as tiling and ceiling battens) the sapwood may be extensive, and its destruction may result in collapse. Replacement of these timbers is the only option available.

# TIMBER DECAY FUNGI:

The fruiting bodies of wood decay fungi vary in size, shape and colour. The type of fungi encountered by pest controllers usually reside in poorly ventilated subfloors, below wet areas of the home, exterior timbers and in areas that retain water in the soil. The durability and type of timbers are factors along with the temperature and environment. Destruction of affected timbers varies with the symptoms involved. Removal of the moisture source usually alleviates the problem. Fungal decay is attractive to termites and if the problem is not rectified it may well lead to future termite attack.

Certificate of Termite Treatment in accordance with AS 3660.2-2000

Terms and Conditions

DISCLAIMER OF LIABILITY TO THIRD PARTIES: - This Certificate is made solely for the benefit of the builder/owner named on the face of this Certificate and no liability or responsibility whatsoever is accepted to any third party who may rely on the Certificate either wholly or in part. Any third party acting or relying on this Certificate in whole or in part does so at their own risk. This disclaimer does not apply to persons responsible for Building Approvals.

1. The effectiveness of this installation is dependent upon the provision of a complete (full) barrier being installed in accordance with AS 3660.2-2000 using approved termiticides, systems and/or products. If the barriers are disturbed, breached or bridged then concealed entry by subterranean termites is possible.

2. No liability is accepted for any failure of a termite barrier and this firm warrants only to provide such remedial action as may be necessary during the first 12 months from the date of this Certificate. No such warranty is provided if there are limitations listed on this Certificate or if the barrier is a partial barrier.

3. The barrier(s) installed, as detailed on this Certificate and in the diagram, provide a barrier against subterranean termites only. The barrier is not a barrier against any other pest(s) and in particular does not provide any barrier against “drywood (KALOTERMITIDAE) or dampwood termites.

4. No responsibility is accepted, or warranty implied, for any termite damage that may occur as the result of termite activity, either past, current or in the future.

5. The termite barrier(s) can be rendered ineffective due to building alterations, renovations, additions (pergolas, awnings, verandahs etc), introducing infested materials, timber offcuts, wood chips and formwork left on site, materials stored against the building. External barriers can be destroyed completely by the installation of lawns, gardens, pathways, landscaping etc adjacent to the building. When making such changes you should first contact this firm. Where such changes are made a further termite barrier installation is essential.

6. Do not use untreated timbers for garden edges or retaining walls. Untreated timber attracts termites.

7. When installing paths, lawn, gardens etc it is very important not to cover air vents or weep holes. If the slab edge is exposed by 75 mm to form part of the termite barrier system then it is equally important not to cover the slab edge unless another form of barrier is installed. Again contact this firm before carrying out any such covering. Where such changes are made a further termite barrier installation is essential.

8. Good ventilation and drainage are important, as poor ventilation and drainage greatly increases the risk of termite attack.

9. This firm takes NO RESPONSIBILITY for the concealed entry by termites resulting from poor building design or poor building practices.

10.  It is the building owner’s responsibility to ensure that the inspections recommended in AS 3660.2-2000 are performed. Please contact this firm.

VERY IMPORTANT

If you become aware of the presence of termites within the grounds or on or within the building you should contact this firm or another termite management firm immediately. You should also notify this firm if you become aware that the installed barrier has been breached or bridged in any way.

The Australian Standard 3660.2-2000 recommends that: - “Regular, competent inspections should be carried out at least on an annual basis but more frequent inspections are strongly recommended”. It also recommends “The building owner should ensure that regular inspections – are carried out by a person competent in Unit 8 Inspect and Report on Timber Pests of the National Pest Management Competency Standards, or equivalent and has experience in accordance with that specified in AS 4349.3-1998”. Termites can build around barriers but can be detected during the recommended inspections.

Modern termiticides have a limited life expectancy. The termite barriers will need to be re-installed. The timing can only be determined by regular, competent inspections as recommended by AS 3660.2-2000 carried out by a qualified termite inspector.

IMPORTANT INFORMATION: The Australian Standard AS 3660.2-2000 Termite Management Part2: In and around existing buildings and structures - Guidelines details “guidelines for the detection and management of subterranean termites active in and around existing buildings and structures.”. The Standard also provides information on “barriers which impede termite from gaining concealed access to  materials in the building”. AS 3660.1-2000, Termite Management Part 1 states that “a termite barrier system constructed in accordance with this Standard cannot prevent termite attack, as barriers may be bridged or breached. Where termites bridge barriers the evidence may be detected during inspections.”

Certificate of Termite Treatment

in accordance with AS 3660.2-2000

Post construction

| Name of |  | MR ACTHISON |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Property Address | 40 SANDRA RD JILLIBY |  |  |  |  |
|  |  |  | Post Code |  |  |

AS 3660.2.2000 Termite Treatment Performed :

Nest Removal, Eradication and Termite Dusting:

| A termite nest located | THE COTTAGE | and |
| --- | --- | --- |

| therefore removed or treated with |  | . All termite |
| --- | --- | --- |

entry points should be found. Finding them may result in us causing damage to your property. We take no responsibility for

the cost of the repair of such damage and you agreed to meet the full cost of all such repairs. Termite entry  points   were

| found in the following area(s) | SLAB EDGE |  |  |
| --- | --- | --- | --- |
| Termite activity was treated by the direct application of |  |  | termiticide dust into termite |

| workings in the following area(s) |  | and will require |
| --- | --- | --- |

inspection  in  20  to 40 days.  If  termite activity  is still present at this inspection then further dusting will be required and a

further inspection will be required.  Such dusting  treatment should continue  until all termite activity has ceased. The liquid termiticide treated zone or barrier should not be installed until after all evidence of termite activity has ceased.

|  | A Treated Zone or Termite Barrier was installed to the following area(s) |  |  |  |  |  |  |  | BATHROOM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EXTERIOR TRENCHING |  |  |  |  |  |  |  |  |  |  | using the liquid |  |  |
|  |  |  | termiticide(s) | TERMIDORE |  |  |  |  |  |  | which contain the active |  |  |  |
|  |  | constituent(s) |  | FRIPINOL |  |  |  | .The concentration of the liquid |  |  |  |  |  |  |
| termiticide/s used was |  |  |  |  | 2.00 | % and the total volume used was | 400 |  |  | Lt. Termiticide barriers degrade (break |  |  |  |  |

down)  over  time and should be  replenished in the future.  The  Termiticide manufacturers claim their products should last

from 2 to 10 years depending on the type and strength of termiticide used and the site conditions. So in the future, after one of the required regular inspections of the property, the inspector may advise you of the need to re-install the treated zone or barrier.

If this treatment was carried out as part of a treatment for active termites then an inspection of the property and buildings should be carried out one to three months after the completion of this treatment and again three months after that. Treatments resulting in partial barriers will require more frequent inspections.

Other Termite Management System (ie reticulation, physical barrier, baiting):

| Another Termite Management System(s) was installed in the following area(s): |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | The System is |  |  |  |  |  |  |
|  | The method(s) of installation was |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | . The system |  |  |  |

integrated with the liquid termiticide barrier and integrated with the building to form the termite barrier.

Please see the section on limitations. Liquid barrier systems should not be installed until after all evidence of

termite activity has ceased (See Sections on Dusting and Baiting above).

The Termite Barrier formed using all or any of the above barriers is a  barrier. Partial barriers are unlikely to be

effective and on going inspections are required and further treatments may also be required. No warranty can be given where only partial barriers have been installed.

Limitations that apply to the above installation are:

ONLY A PARTIAL TREATMENT TO MANAGERS COTTAGE

The treatment will not be complete until all termite activity has ceased and a full Termite Barrier or Treated Zone has been successfully installed. If you become aware of any new activity do not disturb the termites in any way. You should notify this firm of your finds as soon as possible. Please contact us if ever you have any concerns about termites or the effectiveness of the barrier. It is very important that the Termite Barrier is not bridged or breached. DO NOT disturb the treated areas in any way.

A qualified Timber Pest Inspector should inspect the building and its surrounds at least every twelve (12) months. It is strongly recommended by the Australian Standard AS 3660.2-2000 that more frequent inspections (3 to 6 monthly) should be carried out.

| We recommend an inspection every | 12 | Months. |
| --- | --- | --- |

Terms and Conditions on Page 1 form an important part of this Certificate. If the above barrier(s) is/are integrated with the concrete then the concrete forms an integral part of the termite barrier to this structure. In this case, the concrete should have been poured in accordance with AS 3660.1-2000.

This firm did not install any concrete or any part of the building structure that forms any part of the barrier and takes no responsibility for any failure of the Termite Barrier that results from the failure of any concrete or building construction to perform as a Termite Barrier.

| Installation Firm |  | Terence Walsh Pest Management |  |  |  | Name of Installer: |  |  | Terence Walsh |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Firm’s Address: |  |  |  |  |  | Installer’s Licence No |  |  | 1752 |  |  |
|  |  |  | Post code: |  | Insurance Termite Accreditation Number: |  |  |  |  |  | 000568 |
| Telephone | 1300 134 301 |  |  |  | Signature: |  |  | Date: |  | 1 / 6 / 8 |  |

Client or Agent’s Signature: ________________ Date:    /    /

© Copyright RAPID Solutions 2001 - for use by RAPID Solutions clients only

Sketch (Mud map) not to scale: Please indicate North

LEGEND. Termite Activity. Termite Damage. Possible Termite Damage. Areas to be Treated.

| Risk Management Record |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name : MR ACHISON |  |  |  |  |  |  |  | ABN : |  |  |  |  |  |  |
| Address : 40 SANDERS RD JILLIBY |  |  |  |  |  |  |  | PH : |  |  |  |  |  |  |
|  |  |  | Total |  |  |  |  |  | Date |  |  | Order |  |  |
| Product | Concentrate |  | Mix |  | Application |  | Application |  | Last |  |  | Of | Pest |  |
| Name |  |  | Applied |  | Rate |  | Equipment |  | Calibrated |  |  | Treatment | Treated |  |
|  | 900MLS |  | 500LTR |  |  |  | Hand Spray |  | 1 06 08 |  |  |  | Termites |  |
| Nozzle |  | Wind |  | Wind |  | Start |  |  |  | Pump | Protective |  | Label |  |
| Type |  | Direction |  | Speed |  | And | Temp |  |  | Pressure | Clothing |  | & |  |
| Size |  |  |  |  |  | Finish |  |  |  |  | Worn |  | MSDS Read |  |
| 020 |  | NIL |  | NIL |  | 0900 | 12° C |  |  |  | Gloves |  | YES |  |
|  |  |  |  |  |  | 1155 |  |  |  |  | Goggles |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Hat |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Impervious |  |  |  |
|  |  |  |  |  |  |  | HUMIDITY |  |  |  | Boots |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Overalls |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Respirator |  |  |  |
| Risk Assessment : The following were present on or near the site to be treated:- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The Actions taken to eliminate any risk was as follows : - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HOUSE WAS VACATED AFTER TREATMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other Weather Details : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Suite 1, The Pavilion Business Park Pavilion Place, Cardiff NSW Locked Bag 3 Cardiff NSW 2285

RAPID Solutions Pty Ltd ACN 055 149 203

Telephone: (02) 4954 3655

Facsimile: (02) 4954 3660

RAPID Solutions

ABN 49055149203

CERTIFICATE OF CURRENCY

http://www.rapidsolutions.com.au

Email: support@rapidsolutions.com.au

This is intended for use as evidence that the cover summarised below has been approved, in principal and is subject to the

agreement of the premium funding provider. If the premium funding agreement is not approved this policy may be cancelled in accord with the terms of the insurance contracts Act.

Cover shall be subject to all the terms and conditions of the Pacific International Insurance Limited policy document and

schedule.

The Insured:	 Terence Walsh

T/as Terence Walsh Pest Management

PARTICULARS of Insurance COVER

Insurer:

Policy Number:

Period of Insurance Cover:

Limit of Public & Products Liability:

Sub-Limit of Public & Products Liability

Environmental Impairment:

Limit of Professional Indemnity:

Pacific International Insurance Limited

AUS-04-6316

16 AUGUST  2004 To 16 AUGUST 2005

$5,000,000	Any one claim and in the aggregate per

policy period.

$250,000	Any one claim and in the aggregate per

policy period.

$500,000        Any one claim in the aggregate

per policy period.

Issued by RAPID Solutions as agent for Pacific International Insurance Limited.

per..

Initials:- pk-Rev4

RAPID Solutions andlor Pacific International accepts no responsibility for any failure to notify the

recipient of this Certificate of Currency in the event of the policy being cancelled..

Tuesday  17 August  2004

This Treatment was carried out by:

Terence Walsh.

State Licence No: 		1752

TERMITE ACCREDITATION NO:        000568

SIGNED FOR AND ON BEHALF OF:

TERENCE WALSH  PEST MANAGEMENT

Signature:

## Terence Walsh

Pest Management.

Licence No. 1752

ABN – 27 422 482 252

Phone : 1300 134 301

ttermite@bigpond.net.au

Date:	5/6/8

### TO:  MR ACHISON

### TAX Invoice No:

| #### ITEM | COST | GST | TOTAL |
| --- | --- | --- | --- |
| Type Of Service: TERMITE TREATMENT Property: WORKERS COTTAGE 40 SANDRA ST JILLIBY Amount Paid : $1,200.00 |  |  | $1,200.00 $1,200.00 |
| ##### E.& O.E. TERMS: NET DAYS | Total including GST |  | $1,200.00 |

---------------------------------------------------------------------------------------------------------------

Attached is total payment as per Invoice No.:

Client Name:

PLEASE RETURN THIS SLIP WITH YOUR PAYMENT TO:

Cheque for 	$ 0      payable to Terence Walsh Pest Management